DIGITAL FARE METER FOR AUTO-RICKSHAW

MANUFACTURING FEASIBILITY & PROJECT REPORT

Reference Product: PERFECT Model A-786 | 4 Stroke Pulse-1725
By Perfect Engineering, Bangalore

Prepared for: Flavi Dairy Solutions | Kirit Patel | July 2026

# 1. PRODUCT IDENTIFICATION & UNDERSTANDING

| Parameter | Details |
| --- | --- |
| Product Name | Digital Fare Meter / Taximeter for Auto-Rickshaw |
| Reference Brand | PERFECT — Model A-786 / KG-7A-786 ¦ Mfg: Perfect Engineering, Bangalore |
| Key Functions | Fare calculation (Rs/Paise), Distance measurement (Kms), Real-time Clock, Waiting Time |
| Pulse Rate | 1725 pulses/km (tied to vehicle's 4-stroke engine / wheel rotation) |
| Display Type | LED 7-segment — shows Rupees, Paise, Distance, Clock |
| Power Supply | 12V DC from vehicle battery |
| Approval No | Model approval from Legal Metrology Dept (e.g., IND/08/98/171 as seen on unit) |
| Current Market Players | Perfect Engineering (Bangalore), Pricol (Coimbatore), Super Meter, Nimbus Technologies, Precision Electronic Instruments |
| Market Opportunity | New generation SMART meters with GPS + UPI printer + receipt printing replacing old analog/basic digital meters |

# 2. MARKET OPPORTUNITY & DEMAND

| Parameter | Data |
| --- | --- |
| Total Auto-Rickshaws in India (registered) | ~75–80 Lakh (7.5–8 million) — India has world's largest auto fleet |
| Annual New Auto Registrations | ~3–4 Lakh new autos/year + replacement demand |
| Meter Replacement Cycle | Every 3–5 years (mandatory recalibration + failure replacement) |
| Annual Meter Demand (India) | ~8–12 Lakh units/year (new + replacement) |
| Current Market Size | ₹200–300 Crore/year (basic digital meters) |
| Smart Meter Market (GPS + printer) | Emerging — ₹500–800 Crore potential by 2028 |
| Average Meter Selling Price (basic) | ₹1,500–3,000/unit (basic digital) |
| Average Meter Selling Price (smart) | ₹4,000–8,000/unit (GPS + printer + UPI) |
| Key Cities/States | Delhi, Mumbai, Bangalore, Hyderabad, Pune, Chennai, Kolkata, UP, TN, AP, Karnataka, Maharashtra |
| Government Push | Smart City Mission + state transport digitization mandating upgraded meters in metro cities |
| Export Potential | Africa (Nigeria, Kenya, Ghana — tuk-tuks), SE Asia (Bangladesh, Sri Lanka, Nepal, Vietnam) |

# 3. PRODUCT SPECIFICATIONS — BASIC vs SMART MODEL

## 3A. Basic Digital Fare Meter (Model like PERFECT A-786)

| Specification | Details |
| --- | --- |
| Display | 7-segment LED — 5+2 digits (Rs + Paise) + 4 digits (km) |
| Distance Input | Reed switch or Hall effect sensor on wheel — pulse counting |
| Pulse Rate Setting | Configurable (e.g., 1725 pulses/km for 4-stroke autos) |
| Functions | Fare, Distance, Clock, Waiting time, Night charge, Return journey |
| Fare Table | Programmable as per state transport authority notification |
| Operating Temperature | -10°C to +60°C |
| Power | 12V DC ±20%, current < 200mA |
| Tamper Protection | Sealed enclosure, non-resettable meter reading |
| Standards | IS 9000 / IS 16833:2018 ¦ Legal Metrology Act 2009 |
| Enclosure | ABS plastic housing, IP40 minimum |
| Unit Cost (manufacturing) | ₹600–1,200/unit at scale |
| Selling Price (B2B) | ₹1,500–2,500/unit |

## 3B. Smart Digital Fare Meter (Next Generation — Competitive Advantage)

| Feature | Specification | Why It Matters |
| --- | --- | --- |
| GPS Module | SIM868 or u-blox NEO-M8N | Distance via GPS — tamper-proof, accurate |
| GPRS/4G Connectivity | SIM7600 or SIM800 module | Cloud sync, remote monitoring, fleet management |
| Thermal Printer | 58mm thermal printer module | Printed receipt with trip details, QR code |
| UPI / QR Payment | BharatQR / UPI QR display on screen or printed | Cashless payment — mandatory in many cities |
| Touchscreen Display | 3.5" TFT colour touchscreen | Modern UI for driver and passenger |
| Microcontroller | STM32F103 or ESP32 | High performance, low cost |
| RTC | DS3231 — precision clock | Night/day fare, waiting time calculation |
| Panic Button | Emergency SOS to police/family | Safety feature — government encouragement |
| Bluetooth | BLE 5.0 | Smartphone app connectivity for driver app |
| Anti-Tamper | Sealed firmware, encrypted fare table, physical seal | Legal Metrology compliance |
| Power Management | 12V input, 5V/3.3V regulated, battery backup 30 min | Continuous operation |
| Memory | SD card or internal flash — trip log storage | Audit trail, dispute resolution |
| Unit Cost (manufacturing) | ₹2,500–4,000/unit at scale | Premium product |
| Selling Price (B2B/trade) | ₹5,000–8,000/unit | High margin product |

# 4. BILL OF MATERIALS (BOM) — COST PER UNIT

## 4A. Basic Digital Meter (Production volume: 1,000+ units/month)

| Component | Specification | Unit Cost (₹) | Supplier (India) |
| --- | --- | --- | --- |
| Microcontroller | ATmega328P or STM32F030 | 80–150 | DigiKey India, Mouser, Robu.in |
| LED Display (7-segment) | 4+2 modules, common cathode, red | 150–250 | Evelta, Robokits, local electronics |
| Hall Effect Sensor | A3144 or similar for wheel pulse | 15–30 | Electronic Spares, Robokits |
| Real Time Clock IC | DS1307 or DS3231 | 40–70 | Evelta, DigiKey India |
| PCB (custom, 2-layer) | Designed for meter — FR4 board | 80–150 | PCB Power, Circuit Systems India, PCBGOGO |
| ABS Plastic Enclosure | Custom moulded (at scale) or standard box | 100–200 | Local plastics manufacturer, Alibaba India |
| DC-DC Power Module | 12V to 5V/3.3V conversion | 40–80 | Standard module, local |
| Passive Components (R,C,L) | Resistors, capacitors, diodes, transistors | 30–50 | Mouser, DigiKey, local |
| Push Buttons + Reset Switch | Tactile + sealed button | 15–30 | Local electronics market |
| Buzzer + LED indicators | Active buzzer, LEDs | 15–25 | Local electronics market |
| Connectors + Wiring Harness | Automotive grade connectors | 40–80 | Amphenol, Molex India, Weidmuller |
| Firmware (one-time cost) | Written in C, embedded | 0 (amortized) | In-house or outsource ₹1–3 Lakh one-time |
| Assembly Labor | SMT + through-hole + testing | 80–150 | In-house or contract manufacturing |
| Packaging | Box + manual + warranty card | 30–50 | Local |
| TOTAL ESTIMATED COST |  | ₹700–1,300/unit | At 1,000+ units/month scale |

## 4B. Smart Digital Meter — Additional Components

| Additional Component | Specification | Unit Cost (₹) | Notes |
| --- | --- | --- | --- |
| GPS + GPRS Module | SIM868 combo (GPS+GSM+GPRS) | 600–900 | SIMCom distributor India |
| SIM Card Slot + Antenna | Nano SIM + GPS patch antenna | 50–100 | Local electronics |
| Thermal Printer Module | 58mm CSN-A2 or Cashino PTP-II | 400–700 | Aliexpress, local EMS |
| TFT Colour Display | 3.5" ILI9341 SPI | 250–450 | Evelta, Robokits India |
| Bluetooth Module | HC-05 or ESP32 built-in | 80–150 | Robu.in, Evelta |
| Memory (SD card + holder) | Class 10 SD + holder | 60–100 | Local electronics |
| Battery Backup Module | Li-Ion 3.7V 2000mAh + TP4056 charger | 120–200 | Evelta, Robokits |
| Panic Button + GSM Alert | Extra button + firmware feature | 20–40 | Built into same GSM module |
| ADDITIONAL COST TOTAL |  | ₹1,600–2,600 | Smart meter adds ₹1,600–2,600 over basic |
| SMART METER TOTAL COST |  | ₹2,300–3,900/unit | At 500+ units/month scale |

# 5. LICENSES & APPROVALS REQUIRED

| Approval/License | Issuing Authority | Requirement | Process | Cost / Timeline |
| --- | --- | --- | --- | --- |
| Model Approval (Taximeter) | Directorate of Legal Metrology, Dept of Consumer Affairs, GOI | MANDATORY — No meter can be sold without this | Submit 3–5 samples + tech docs + IS compliance report to Central LM HQ, New Delhi | ₹5,000–20,000 fee ¦ 3–6 months |
| Manufacturer's License | State Legal Metrology Controller (Karnataka/Gujarat/your state) | MANDATORY — required before production | Factory inspection + application on LMOMS portal + fee payment | ₹2,000–10,000 ¦ 1–3 months |
| BIS Certification (IS 16833:2018) | Bureau of Indian Standards | Mandatory for electronic fare meters | Product testing at BIS-accredited lab + factory audit + BIS mark license | ₹50,000–2 Lakh ¦ 3–6 months |
| MSME Registration (Udyam) | Ministry of MSME (online) | Recommended for scheme benefits | Online registration at udyamregistration.gov.in — free | Free ¦ 1 day |
| Factory License | State Factory Inspector (Factories Act) | Required if >10 workers with power | Local factory inspector application | ₹5,000–20,000 ¦ 1 month |
| GST Registration | GST Portal | Mandatory for sales >₹20L/yr | Online at gst.gov.in | Free ¦ 3–7 days |
| State Transport Approval | State Road Transport Authority | Each state may require separate approval for use in their autos | Submit model approval + test data to state RTO/transport authority | Varies by state ¦ 1–3 months |
| EMI/EMC Testing | ERTL or NABL-accredited lab | For BIS and export compliance | Send sample for electromagnetic interference testing | ₹50,000–1,50,000 ¦ 4–8 weeks |

Key Reference Laws: Legal Metrology Act 2009 | Legal Metrology (Approval of Models) Rules 2011 | IS 9000 / IS 16833:2018

# 6. MANUFACTURING SETUP — INVESTMENT PLAN

## 6A. Small-Scale Setup (500–1,000 units/month)

| Item | Requirement | Cost (₹) |
| --- | --- | --- |
| Factory Space | 1,000–1,500 sq ft industrial unit | ₹30,000–60,000/month rent OR buy ₹50–80L |
| PCB SMT Assembly Equipment | Stencil printer + reflow oven + pick & place (semi-auto) | ₹10–20 Lakh |
| Soldering Station + Tools | Rework stations, hand soldering irons, flux stations | ₹2–4 Lakh |
| Testing Equipment | Oscilloscope, multimeter, power supply, functional test jig | ₹3–5 Lakh |
| Programming Setup | MCU programmers (AVR ISP, STLink), laptop/PC, firmware tools | ₹50,000–1 Lakh |
| Calibration Standard | Reference distance simulator + calibration weights jig | ₹1–2 Lakh |
| ESD Protection Setup | ESD mat, wrist straps, antistatic bags | ₹50,000–1 Lakh |
| PCB Design Software License | KiCAD (free) or Altium ($2K/yr) | ₹0 (KiCAD) – ₹1.5 Lakh/yr |
| Firmware Development | In-house engineer or outsource | ₹2–5 Lakh one-time |
| Working Capital (3 months BOM) | Components + packaging stock | ₹15–30 Lakh |
| Regulatory/Approvals Costs | BIS + Legal Metrology model approval + testing labs | ₹5–10 Lakh |
| TOTAL CAPEX (Small Scale) |  | ₹40–80 Lakh |

## 6B. Medium-Scale Setup (3,000–5,000 units/month)

| Item | Cost (₹) |
| --- | --- |
| Fully automatic SMT line (pick & place + reflow + AOI) | ₹40–80 Lakh |
| Factory (3,000–5,000 sq ft) | ₹80L–1.5 Cr (own) OR ₹60K–1 L/month rent |
| Custom injection moulding tool for enclosure | ₹10–20 Lakh (one-time mould cost) |
| In-house PCB fabrication (optional) | ₹50–80 Lakh (or outsource PCB fab) |
| Quality lab + test instruments | ₹10–15 Lakh |
| Working Capital (3 months) | ₹50–80 Lakh |
| Certifications + approvals | ₹10–15 Lakh |
| IT + ERP + software | ₹5–10 Lakh |
| TOTAL CAPEX (Medium Scale) | ₹2.5–5 Crore |

# 7. FINANCIAL PROJECTIONS

| Scenario | Volume/Month | Revenue/Month | Revenue/Year | Gross Margin | Net Profit/Year |
| --- | --- | --- | --- | --- | --- |
| Small Scale — Basic Meter | 1,000 units @ ₹2,000 avg selling | ₹20 Lakh | ₹2.4 Crore | 35–40% | ₹60–80 Lakh/yr |
| Small Scale — Smart Meter Mix | 500 basic + 200 smart (@₹6,000) | ₹22 Lakh | ₹2.64 Crore | 40–45% | ₹80–100 Lakh/yr |
| Medium Scale — Mixed | 3,000 basic + 1,000 smart | ₹1.2 Crore | ₹14.4 Crore | 42–48% | ₹4–6 Crore/yr |
| Large Scale + Export | 10,000 units (India + Africa/SE Asia) | ₹3–4 Crore | ₹36–48 Crore | 45–50% | ₹15–20 Crore/yr |

# 8. EXISTING MANUFACTURERS — COMPETITION ANALYSIS

| Company | Location | Products | Strength | Weakness |
| --- | --- | --- | --- | --- |
| Perfect Engineering | Bangalore, Karnataka | Basic digital meters (PERFECT brand) | Established brand, Legal Metrology approved, large installed base | Old technology, no GPS/smart features |
| Pricol Limited (listed company) | Coimbatore, Tamil Nadu | Automotive instruments, speedometers, fare meters | Large scale, R&D, national reach, export | Premium pricing, B2B focused |
| Nimbus Technologies | Mumbai | Digital fare meters, GPS meters | GPS-enabled models available | Limited distribution outside Maharashtra |
| Super Meter | Delhi/NCR | Basic and semi-smart meters | Strong North India presence | No receipt printer, basic features |
| Precision Electronic Instruments | Mumbai | Electronic meters for transport | Legacy approvals | Outdated design |
| YOUR OPPORTUNITY | — | Smart meter: GPS + printer + UPI QR + safety | First mover in fully integrated smart meter at affordable price | New entrant — need to build approval + channel |

# 9. COMPETITIVE STRATEGY — HOW TO WIN

1. SMART METER FIRST MOVER: Launch India's most affordable GPS + UPI QR + printer fare meter in ₹5,000–6,000 range — undercut smart meter competitors

2. BUNDLED FLEET SOLUTION: Offer meter + cloud app + fleet management dashboard for auto aggregators (Ola, Rapido, Uber Auto, city fleet operators)

3. STATE TRANSPORT TENDERS: Bid for state transport authority mandated "smart meter upgrade" tenders — Karnataka, Maharashtra, Delhi are actively pushing smart meters

4. AUTHORIZED SERVICE NETWORK: Establish service + calibration centers across top 10 cities — differentiator as established players have weak after-sales

5. EXPORT TO AFRICA + SE ASIA: Nigeria, Kenya, Ghana (tuk-tuks), Bangladesh, Sri Lanka, Nepal — offer basic meter at $20–30 USD — massive volume

6. DRIVER APP INTEGRATION: Develop companion Android app for drivers — shows trip history, earnings, GPS route, UPI settlements — high stickiness

7. OEM for AUTO MANUFACTURERS: Approach Bajaj Auto, TVS, Piaggio for factory-fit meter supply — potential 50,000+ units/year

# 10. STEP-BY-STEP LAUNCH PLAN

| Phase | Duration | Activities | Budget |
| --- | --- | --- | --- |
| Phase 1 — Design & Prototype | 0–3 months | Hire electronics engineer; design PCB; write firmware; build 10 prototypes; test in real auto | ₹5–10 Lakh |
| Phase 2 — Approvals | 3–9 months | Submit for Legal Metrology model approval; BIS testing; IS 16833:2018 compliance; manufacturer license | ₹8–15 Lakh |
| Phase 3 — Pilot Production | 9–12 months | Set up small assembly unit; produce 200–500 units; sell to select auto drivers + fleet operators for feedback | ₹20–30 Lakh |
| Phase 4 — Scale Up | 12–24 months | Invest in SMT line; scale to 1,000–3,000 units/month; build distributor network; bid for state tenders | ₹50–80 Lakh |
| Phase 5 — Smart Meter Launch | 18–30 months | Launch GPS + printer + UPI smart meter; approach state transport bodies; OEM tie-ups; export market entry | ₹1–2 Crore |

# 11. KEY VENDORS & TECHNOLOGY PARTNERS

| Need | Vendor / Resource | Contact / Link |
| --- | --- | --- |
| PCB Design (outsource) | KELS PCB, Embitel, Softedge Systems | Local EMS companies, Upwork for PCB design freelancers |
| PCB Fabrication | PCB Power Market (Gandhinagar, Gujarat!) | pcbpower.com ¦ +91-79-4003-4040 — GUJARAT BASED |
| SMT Contract Manufacturing | SFO Technologies (Kochi), Kaynes (Mysore), NEL (Bangalore) | Contract until own line is ready |
| GPS/GPRS Module | SIMCom Wireless Solutions (through Rajguru Electronics) | rajguruelectronics.com |
| Thermal Printer Module | Cashino Technology (via Alibaba India) | cashino.com.cn or Alibaba B2B |
| Microcontrollers | Element14 India / DigiKey India / Mouser India | in.element14.com ¦ digikey.in |
| Enclosure Moulding | Local plastics injection moulding company (Gujarat — many in Ahmedabad) | Industrial area Ahmedabad/Rajkot |
| Legal Metrology Approval | Directorate of Legal Metrology, New Delhi | lm.doca.gov.in ¦ +91-11-23378797 |
| BIS Certification | Bureau of Indian Standards, New Delhi | bis.gov.in ¦ ISI certification division |
| EMI/EMC Testing | ERTL (West) Mumbai or ERTL (South) Bangalore | ertlmumbai.gov.in ¦ ertlblr.gov.in |
| Electronics Engineer (hire) | Naukri.com, LinkedIn — firmware + embedded systems | Search: "Embedded Systems Engineer Fare Meter" Bangalore/Gujarat |

# 12. SYNERGY WITH FLAVI DAIRY SOLUTIONS

How this business connects to Flavi's strengths:

MANUFACTURING EXPERTISE: Flavi designs food/dairy processing plants — same project management, EPC, and manufacturing setup skills apply to electronics manufacturing plant design

KRISHNA ALLIED INDUSTRIES (Kalol, Gujarat): Already in equipment manufacturing — the electronics division can be a sister unit using same factory infrastructure

GUJARAT ADVANTAGE: PCB Power Market (Gandhinagar), electronics component suppliers (Ahmedabad), moulding companies (Rajkot) — all in Gujarat

FOOD INDUSTRY IOT APPLICATION: The same GPS + IoT + sensor expertise in fare meters can be applied to dairy plant IoT monitoring — a natural product extension

EXPORT CAPABILITY: Existing Flavi international consulting network (Africa dairy projects) = ready channel for fare meter export to African tuk-tuk markets

GOVERNMENT CONNECTIONS: Flavi's FSSAI/govt compliance experience helps navigate Legal Metrology approval process faster

Prepared by: Flavi Dairy Solutions AI Research System | July 2026 | Confidential Business Report

Sources: Legal Metrology Act 2009, BIS, APEDA, DigiKey India, PCB Power, SIMCom, Directorate of Legal Metrology, ERTL, industry estimates