WORKSTATION SPEC ANALYSIS REPORT

Lenovo ThinkStation P620 | AMD Ryzen Threadripper PRO 3945WX

Prepared by: Flavi Dairy Solutions | July 2026

# 1. MACHINE SPECIFICATIONS (AS LISTED)

| Component | Specification | Actual / Verified |
| --- | --- | --- |
| Brand / Model | Lenovo ThinkStation P620 | ✅ Correct |
| Processor | AMD Ryzen Threadripper PRO 3945WX | ✅ Correct |
| Cores / Threads | 12 Core / 14 Thread (listed) | ⚠️ WRONG — Actual: 12C / 24T |
| Base / Boost Clock | Not listed | 4.0 GHz base / 4.3 GHz boost |
| TDP | Not listed | 280W (High-performance workstation) |
| RAM | 32 GB DDR4 ECC | ✅ ECC RAM — workstation grade |
| Storage | 1 TB NVMe SSD | ✅ Fast PCIe NVMe |
| GPU | 4 GB AMD Graphics | ⚠️ Low VRAM — entry workstation GPU |
| Platform | WRX80 Chipset | ✅ 8-channel ECC, 128 PCIe lanes |
| Form Factor | Tower Workstation | ✅ Full tower |
| Launch Year | ~2020 | 4–5 year old platform |

# 2. SPEC ERROR — IMPORTANT ⚠️

LISTING ERROR FOUND: The machine is advertised as "14 Thread" which is factually INCORRECT.

The AMD Ryzen Threadripper PRO 3945WX has 12 Cores with 2 threads per core (Simultaneous Multi-Threading) = 24 Threads total. If any seller is advertising 14 threads, it is either a typo or misrepresentation. Verify using CPU-Z software before finalizing purchase.

# 3. COMPONENT-WISE VERDICT

| Component | What You Get | Verdict | Notes |
| --- | --- | --- | --- |
| CPU: 3945WX | 12C/24T, 4.3GHz Boost | ✅ Excellent | Top workstation CPU, overkill for basic IT |
| RAM: 32GB ECC | DDR4 ECC Registered | ⚠️ Entry Level for this platform | Platform supports up to 2TB — 32GB is minimum config |
| Storage: 1TB NVMe | PCIe Gen 4 NVMe | ✅ Adequate | Fast, expandable with additional drives |
| GPU: 4GB AMD | Likely AMD Pro W-series | ⚠️ Weak for GPU compute | Fine for display/CAD; weak for AI/ML/rendering |
| Platform: WRX80 | 8-ch ECC, 128 PCIe lanes | ✅ Future-proof | Can add more RAM, GPUs, NVMe drives |

# 4. USE CASE FIT FOR IT BUSINESS

## 4A. IDEAL USE CASES (Buy it!)

✅ Running 10–20+ Virtual Machines simultaneously (VMware/Hyper-V)

✅ Heavy software development and code compilation

✅ Server simulation and cloud infrastructure testing

✅ Database server work (SQL Server, Oracle, PostgreSQL)

✅ CAD/CAM and 3D modelling (upgrade GPU recommended)

✅ Video editing and post-production rendering

✅ Network lab with multiple VMs (Cisco, Linux, Windows Server)

## 4B. OVERKILL USE CASES (Save your money)

❌ Office work, emails, web browsing — Core i5 laptop sufficient

❌ Basic networking / IT support / helpdesk work

❌ Light web development (HTML, CSS, PHP, Python scripts)

❌ General business computing

## 4C. UNDERPOWERED FOR (Needs upgrade)

❌ AI / Machine Learning training → Need 16GB+ GPU (RTX 4080/4090)

❌ Heavy GPU rendering (Blender, 3ds Max) → 4GB VRAM insufficient

❌ Latest gaming / modern DirectX 12 graphics

# 5. MARKET PRICE ESTIMATE (2026)

| Market | Condition | Price (INR) | Price (USD) |
| --- | --- | --- | --- |
| India — New (if available) | New | ₹3,50,000 – ₹5,50,000 | $4,000 – $6,500 |
| India — Refurbished | Refurb | ₹1,20,000 – ₹2,00,000 | $1,400 – $2,400 |
| India — Used/Grey Market | Used | ₹80,000 – ₹1,40,000 | $950 – $1,650 |
| USA — Refurbished | Refurb | N/A | $900 – $1,600 |
| USA — New (if available) | New | N/A | $4,500 – $7,000 |

💡 Value Tip: This machine launched in 2020 and is now 4–5 years old. At FULL price it is overpriced for 2026. At ₹80,000–₹1,20,000 (used/refurb) it represents GOOD VALUE for heavy workstation use.

# 6. FINAL RECOMMENDATION

| Your IT Business Type | Recommendation |
| --- | --- |
| Heavy VM / Dev / Server simulation | ✅ BUY at ₹80K–₹1.2L (used/refurb) — excellent value |
| General IT support / networking / helpdesk | ❌ SKIP — buy Dell/HP Core i7 laptop at ₹60K–₹80K |
| AI / ML / Data Science work | ❌ SKIP — get RTX 4070/4080 GPU machine instead |
| CAD / 3D / Architecture + upgrade GPU | ✅ GOOD — upgrade GPU to Quadro/WX7100 separately |
| Cloud infra testing / DevOps lab | ✅ EXCELLENT — 24 threads ideal for multi-VM labs |

# 7. CHECKLIST BEFORE BUYING

□ Get original invoice / warranty card from seller

□ Verify CPU using CPU-Z software — confirm 12C / 24T

□ Check RAM slots used (8 slots available — confirm expandability)

□ Run CrystalDiskInfo to check NVMe SSD health

□ Check GPU model number (run GPU-Z) — confirm 4GB VRAM

□ Boot into BIOS — check no throttling/power limits set

□ Run Prime95 stress test for 10 min — check for thermal issues

□ Ask if thermal paste has been freshly applied (5-yr old machine)

□ Negotiate price — this is a 2020 machine, push for best rate

□ Ensure Windows 11 Pro license included OR Linux-ready

Prepared by Flavi Dairy Solutions | Kirit Patel | July 2026